#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#define SENSOR_PIN  34  // ESP32 Analog Input Pin

#define MOTOR_IN1   14  // Connected to IN1 on MX1508 driver

#define MOTOR_IN2   27  // Connected to IN2 on MX1508 driver (or tie IN2 permanently to GND)

// Adjust these values after observing Serial Monitor readings for dry vs wet soil

const int DRY_VALUE = 3200;  // Typical reading in dry air (approx 3000 - 4095)

const int WET_VALUE = 1200;  // Typical reading when submerged in water

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(115200);

delay(500);

Serial.println("\n--- Soil Moisture System with MX1508 ---");

// ESP32 default I2C pins: SDA = GPIO21, SCL = GPIO22

Wire.begin(21, 22);

lcd.init();

lcd.backlight();

pinMode(MOTOR_IN1, OUTPUT);

pinMode(MOTOR_IN2, OUTPUT);

// Initialize Motor OFF

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, LOW);

lcd.setCursor(0, 0);

lcd.print("System Loading..");

for (int a = 0; a < 16; a++) {

lcd.setCursor(a, 1);

lcd.print(".");

delay(100);

}

lcd.clear();

Serial.println("System Ready.");

}

void soilMoisture() {

int rawValue = analogRead(SENSOR_PIN);

// Map raw analog reading to percentage based on calibration values

int percentage = map(rawValue, DRY_VALUE, WET_VALUE, 0, 100);

percentage = constrain(percentage, 0, 100);

// Debug output via Serial Monitor

Serial.print("[DEBUG] Raw ADC: ");

Serial.print(rawValue);

Serial.print(" | Moisture: ");

Serial.print(percentage);

Serial.println("%");

// Display on LCD

lcd.setCursor(0, 0);

lcd.print("Moisture: ");

lcd.print(percentage);

lcd.print("%   ");  // Extra spaces clear previous double-digit values

// MX1508 Motor Control Logic

lcd.setCursor(0, 1);

if (percentage < 30) {

// Turn Motor ON

digitalWrite(MOTOR_IN1, HIGH);

digitalWrite(MOTOR_IN2, LOW);

lcd.print("Motor: ON       ");

Serial.println("[MOTOR] State: ON (Low Moisture)");

} else {

// Turn Motor OFF

digitalWrite(MOTOR_IN1, LOW);

digitalWrite(MOTOR_IN2, LOW);

lcd.print("Motor: OFF      ");

Serial.println("[MOTOR] State: OFF (Sufficient Moisture)");

}

}

void loop() {

soilMoisture();

delay(1000);  // Sample every second

}